注意事项：ddl 2025.11.14，直接上传电子版至学在浙大

Problem 1

In a direct-mapped cache, each memory address is associated with one possible block within the cache. The memory address is divided into three fields, as shown below.

| Tag | Index | (Byte) Offset |
| --- | --- | --- |

Suppose we have a 128Bytes of data in a direct-mapped cache with 16-byte blocks. Determine the size of the Tag, Index and (Byte) Offset fields if the memory address is 32-bit.

Problem 2

For a direct-mapped cache design with a 32bit address, the following bits of the address are used to access the cache.

| Tag | Index | Offset |
| --- | --- | --- |
| 31-13 | 12-6 | 5-0 |

- What is the cache line size(in words)?

- How many entries does the cache have?

- What is the ratio between total bits required for such a cache implementation over the data storage bits? (Note the structure of the direct-mapped cache! Don’t forget the valid bit.)

Problem 3

Cache is important to providing a high-performance memory hierarchy to processors. Below is a list of 32-bit memory address references, given as word address.

3, 180, 43, 2, 181, 88, 164, 180, 89.

- For each of these references, identify the binary address, the tag, and the index given a direct-mapped cache with 8 two-word blocks. Also list if each reference is a hit or a miss, assuming the cache is initially empty.

| Word address | Binary address | Tag | Index | Hit/Miss |
| --- | --- | --- | --- | --- |
| 3 | 0000 0011 |  |  |  |
| 180 | 1011 0100 |  |  |  |
| 43 |  |  |  |  |
| 2 |  |  |  |  |
| 181 |  |  |  |  |
| 88 |  |  |  |  |
| 164 |  |  |  |  |
| 180 |  |  |  |  |
| 89 |  |  |  |  |

- For each of these references, identify the binary address, the tag, and the index given a direct-mapped cache with 4 four-word blocks. Also list if each reference is a hit or a miss, assuming the cache is initially empty.

| Word address | Binary address | Tag | Index | Hit/Miss |
| --- | --- | --- | --- | --- |
| 3 | 0000 0011 |  |  |  |
| 180 | 1011 0100 |  |  |  |
| 43 |  |  |  |  |
| 2 |  |  |  |  |
| 181 |  |  |  |  |
| 88 |  |  |  |  |
| 164 |  |  |  |  |
| 180 |  |  |  |  |
| 89 |  |  |  |  |

Problem 4

This exercise examines the impact of cache designs. For this exercise, the table of word address streams is shown in below.

1, 134, 212, 1, 135, 213, 162, 161, 2, 44, 41, 221

What is the miss rate for a fully associative cache with two-word blocks and a total size of 8 words, using LRU replacement?

Problem 5

You are trying to reverse-engineer the characteristics of a cache in a system, so that you can design a more efficient, machine-specific implementation of an algorithm you are working on. To do so, you have come up with three patterns that access various bytes in the system in an attempt to determine the following four cache characteristics:

• Cache block size (8, 16, 32, 64, or 128 B)

• Cache associativity (1-, 2-, 4-, or 8-way)

• Cache size (4 or 8 KB)

• Cache replacement policy (LRU or FIFO)

However, the only statistic that you can collect on this system is cache hit rate after performing the

access pattern. Here is what you observe (the address is in bytes):

Assume that the cache is initially empty at the beginning of the first sequence, but not at the

beginning of the second and third sequences. The sequences are executed back-to-back, i.e., no other

accesses take place between the three sequences. Thus, at the beginning of the second (third)

sequence, the contents are the same as at the end of the first (second) sequence.

Based on what you observe, what are the following characteristics of the cache? Explain the reason.

- Cache block size (8, 16, 32, 64, or 128 B)?

- Cache associativity (1-, 2-, 4-, or 8-way)?

- Cache size (4 or 8 KB)?

- Cache replacement policy (LRU or FIFO)?